PREPARATIONS

- PHYSICAL PROJECT EXECUTION READINESS CHECK.

WHAT

A site visit is a physical inspection or survey of the prospective site being considered for forest creation.

WHY

A site visit helps in determining feasibility and scope of being able to plant a forest on the prospective site. It will help the creators of the forest assess the prospective site on key parameters that need to be fulfilled under pre project preparations.

WHO

The site should first be visited by the owners of the land. This sometimes becomes important when the owners have been living away from the site and are not aware of actual conditions on ground. For e.g. the approach road to the site might be blocked and needs to be fixed before planning afforestation. The site should then be visited by the forest creators.

WHEN

This is the very first step.

HOW

- Go to the site and take pictures of the proposed afforestation area.

- Check the site for Afforestation Readiness by confirming the availability of.

- Fencing

- Maintenance Staff

- Running Water

- Sunlight

- DEBRIS AND WEEDS CLEANING OF THE ENTIRE AREA

WHAT

The property selected for afforstation should not have wild grass, weed etc. growing. If there is any such growth then it should be cleared

WHY

A site with wild weed and grass growing everywhere will have the following negative impacts:

- The weed/grass will spread on to the forest and jeopardize the health of the forest

- Storage and movement of material will become difficult

- People working on-site (workers and machine drivers) are prone to injuries by thorns etc.

WHO

Labor should be hired for this or the area could be voluntarily cleared. If the site is huge, then a JCB or a John Deere tractor’s bucket is also effective.

WHEN

Essential part of pre execution area preparation

HOW

Clearing out should be done manually or with machine. Remember the following:

- The weeds being pulled out should be thrown away from the site or else it tends to re-grow.

- Weed growth is not greenery. Weeds take away nutrition from the soil. Soil needs a proper forest cover, for it to remain healthy and nutritious. Wild undergrowth in a forest should not be confused with wild weed growing across a property that has been left unchecked.

ILLUSTRATIONS/PHOTOS/ANNEXURES

- WATERING FACILITY INSTALLATION

WHAT

Plumbing work for watering the forest.

WHY

To water the forest everyday for 2-3 years.

WHO

Professional Plumbers

WHEN

Immediately after the master plan is finalized.

HOW

The forest needs to be watered the old fashioned way using a hosepipe with a shower. Hence, watering has to be done manually. Do NOT try drip irrigation, sprinklers etc.

ILLUSTRATIONS/PHOTOS/ANNEXURES

- PHYSICAL DEMARCATION OF AREAS

WHAT

Marking of the actual afforestation area as per the master plan.

WHY

For the machinery to begin the earthwork, it needs to be given an exact area to work on.

WHO

Project Execution team.

WHEN

Before the start of earthwork.

HOW

Ensure that the area being marked matches 100% with the master plan.

Ensure that you have the CAD drawings with you during the physical demarcation exercise. The CAD drawings will give the exact dimensions and mound designs.

A simple way of physically demarcating is with limestone powder. However you might want to look into options such as wooden pegs and ropes used in combination. Measurement is the key and hence having a good measuring tape is very important. Make sure every measurement taken during the demarcation exercise, matches with the AutoCAD drawings. Typically, it takes 3-4 people to measure and demarcate effectively.

Once demarcated, always double check and verify that the markings and measurements are right.

Mark all the areas drafted on afforestation Execution design.

ILLUSTRATIONS/PHOTOS/ANNEXURES

- INSTALLING AREA IDENTIFICATION BOARDS FOR THE SITE

WHAT

There are several important areas on site, apart from the one where the actual forest is being created. We must install identification and direction boards/signage for the same.

WHY

It is easy to feel lost when one is looking for a specific area on the property during forest creation. These signages will help people save time.

WHO

Signages should be made in-house by the owners. Installation should be done by the creators. Check illustration as example.

WHEN

Essential part of pre execution area preparation

HOW

Get the following signages made and installed:

- Arrows

- Sapling storage area

- Biomass storage area

- Mulch Storage area

- Other Consumables

- Tools and equipment storage area

- Toilets

- Resting Sheds

- Drinking water and refreshments

- Meeting space

- Forest

Please check the CAD drawings and do a physical check before installing the boards.

ILLUSTRATIONS/PHOTOS/ANNEXURES

- MAKING APPROACH ROAD TO AND FROM MARKED AREAS

WHAT

Making roads and pathways as per the execution drawing.

WHY

To move materials, people and vehicles in and around site.

WHO

Earthmovers

WHEN

Before material procurement.

HOW

On sites that have been lying vacant for too long, path making to and from the unloading area is only about clearing out all the weed growth. Additionally, one needs to keep in mind that trucks and tractors have enough space on the paths to drive. If the path is just earth/soil, then make sure that it does not have big rock boulders and stones. This ensures safety. The path could be gravel, sand, soil, stone paved or tar.

ILLUSTRATIONS/PHOTOS/ANNEXURES

- MOUND IDENTIFICATION BOARDS ON SITE

WHAT

As explained earlier, forest creation happens on mounds. Once the earthwork is done, what we create is a ‘mound’ on which the actual plantation happens. Each mound to be dug up and planted should have a serial number.

WHY

A unique serial number given to each mound will help in the following ways:

- It will give us the order in which the areas demarcated need to be dug out

- It gives us the order in which the mounds are created and planted upon. This gives the exact age of each mound. This helps in tracking growth and survival with start date indicated by the number given to the mound.

WHO

Signages should be made in-house by the owners. Installation should be done by the creators. Check illustration as example

WHEN

Essential part of pre execution area preparation

HOW

Here’s how this needs to be done:

- Each mound typically is 100 square meters. Once demarcated for digging, it should have a board/signage next to it with its serial number.

- This is the order that needs to be followed in earthwork and plantation.

- Remember that when the earth is dug out, all the soil is dumped next to the trench. Do not start digging work in front of this trench, unless the plantation for it completes. Any digging in front will block access to the original trench.  Thus, in this case, digging work for the second mound should begin only when the first one has been planted.

- One must remember that the order in which the earthwork and planting needs to happen, should be fixed keeping in mind easy accessibility and free working area for each mound.

ILLUSTRATIONS/PHOTOS/ANNEXURES